Test of Step-up Converter Pulse

STEP-UP Voltage Control PWM

Condition

- Control the voltage value by adjusting the pulse width.

Description

STEP UP VOLTAGE Description

- T1: Step-up voltage rising time

- T2: Voltage keeping Time

- T3: Step-up voltage falling time

T2 Description

- A: Upper Range Value >> 42.0v

- B: Target Value >> 40.0v

- C: Lower Range Value >> 38.0v

- EFFECT RANGE: 1.4v

Test Conditions

Target

- Search how to implement STEP-UP Voltage Control PWM.

- Compare with features of implementation methods

Hardware Conditions

- STM32 NUCLEO-L412KB. MCU model is STM32L412KBU6

- System clock is 80MHz, timer counter resolution is 1 MHz (prescaler is 80)

Software Conditions

- STM32CubeIDE Version: 1.10.1

References

- RM0394 Reference manual (STM32L41xxx/42xxx/43xxx/44xxx/45xxx/46xxx advanced Arm®-based 32-bit MCUs)

- STM32L412xx datasheet

- STM32 NUCLEO-L412RB-P schematic

- User manual STM32 NUCLEO-L412RB-P

Method 1

Concepts

- PWM1: TIM2 CH1

- OC2: Output compare of TIM2 CH2. Pulse could not be pulse in really.

- PWM2: TIM1 CH1

- TIM1 and TIM2 are synchronized timers, TIM2 is master and TIM1 is slave. Trigger of TIM1 start is OC2 matching

Description

- TIM1 and TIM2 are independent

- PWM1 and PWM2 are configure as the same conditions (frequency and duty ratio)

- Matching time of OC2 is w1 + d

- Delay is handled by HW

Comparison of Methods

Pros and Cons

| Method | Advantage | Disadvantage |
| --- | --- | --- |
| 1 | 1. Controlled by HW | 1. Settings are complex and not intuitive 2. Pulse width and delay cannot be changed during PMWs run. |
| 2 | 1. Concept is simple | 1. Pulse width and delay cannot be changed during PMWs run. 2. Controlled by SW, so process delay must be considered |
| 3 | 1. Pulse width or delay can be changed during PMWs run. 2. Controlled by SW, but process delay can be ignored | 1. Concept is complex and not intuitive 2. Deep consideration is required to handle process 3. Two interrupts are required every period of PWM |
| 4 | 1. Pulse width or delay can be changed during PMWs run. | 1. Concept is complex and not intuitive 2. Deep consideration is required to handle process |

Resource Required

| Method | Resources Required |
| --- | --- |
| 1 | 2 Timers and 2 GPIOs |
| 2 | 2 Timers and 2 GPIOs |
| 3 | 1 Timer and 2 GPIOs |
| 4 | 1 Timer and 2 GPIOs |

Process Delay Time and Power Consumption

| Method | Process Delay (us) | Power Consumption (AVG mA) |
| --- | --- | --- |
| 1 | 0 | 20.38 |
| 2 | 2.5 | 18.03 |
| 3 | 0 | 19.92 |
| 4 | 0 | 19.91 |

※ 1 LED is used

Reference – Method 1

Reference – Method 2

Reference – Method 3

Reference – Method 4