Test of Stimulation Pulse

Delay between Two PWMs

Condition

- Two PWMs have the same frequency and pulse width

Description

- p1 = p2 and w1 = w2

- Pulse of PWM 1 is output and after delay, d, pulse of PWM2 is output

- w1(or w2) + d < p1(or p2)/2, so w1+d is less than duty ratio 50% of PWM1 (or PWM2)

Test Conditions

Target

- Search how to implement delay between two PWMs

- Compare with features of implementation methods

Hardware Conditions

- STM32 NUCLEO-L412RB-P. MCU model is STM32L412RBP

- System clock is 80MHz, timer counter resolution is 1 MHz (prescaler is 80)

- PWM frequency is 100Hz (period is 10ms), and duty ratio is 10 % (pulse width is 1 ms)

Software Conditions

- STM32CubeIDE Version: 1.10.1

References

- RM0394 Reference manual (STM32L41xxx/42xxx/43xxx/44xxx/45xxx/46xxx advanced Arm®-based 32-bit MCUs)

- STM32L412xx datasheet

- STM32 NUCLEO-L412RB-P schematic

- User manual STM32 NUCLEO-L412RB-P

Method 1

Concepts

- PWM1: TIM2 CH1

- OC2: Output compare of TIM2 CH2. Pulse could not be pulse in really.

- PWM2: TIM1 CH1

- TIM1 and TIM2 are synchronized timers, TIM2 is master and TIM1 is slave. Trigger of TIM1 start is OC2 matching

Description

- TIM1 and TIM2 are independent

- PWM1 and PWM2 are configure as the same conditions (frequency and duty ratio)

- Matching time of OC2 is w1 + d

- Delay is handled by HW

Method 2

Concepts

- PWM1: TIM2 CH1

- OC2: Output compare of TIM2 CH2. Pulse could not be pulse in really.

- PWM2: TIM1 CH1

- Start PWM2 in interrupt handler of TIM1 start is OC2 matching

Description

- TIM1 and TIM2 are independent

- PWM1 and PWM2 are configure as the same conditions (frequency and duty ratio)

- Matching time of OC2 is w1 + d

- Delay is handled by SW

Method 3

Concepts

- PWM1: TIM2 CH1

- OC2 and PWM2: TIM2 CH1

- Compare mode of OC2 is toggle mode

- OC2 is used as PWM2. OC2 is not PWM accurately but PWM pulses are output by SW

Description

- TIM2 is used only. and OC2 interrupt is used

- When start (Ps), OC2 match time is w1+d.

- At first matching, OC2 match time is w2+d, P2.

- At second matching, OC2 match time is w1+d, P1. After update event, TIM2 counter is start from 0, so first matching event occurs and next second matching.

Method 4

Concepts

- Concepts and configurations are the same as Method 3. But DMA is used instead interrupt to handle OC2 matchings

- Current source pulses are addde

- Pulse width of an_ctrl and ca_ctrl is pw + 2s. And d ≥ 2s

- Pulse width of cur_ctrl is pw, and not PWM pulse type.

- Timings

| Points | Timing |
| --- | --- |
| S | 0 |
| T1 | s |
| T2 | pw+s |
| T3 | pw+d |
| T4 | pw+d+s |
| T5 | 2pw+d+s |
| T6 | 2pw+d+2s |

Comparison of Methods

Pros and Cons

| Method | Advantage | Disadvantage |
| --- | --- | --- |
| 1 | 1. Controlled by HW | 1. Settings are complex and not intuitive 2. Pulse width and delay cannot be changed during PMWs run. |
| 2 | 1. Concept is simple | 1. Pulse width and delay cannot be changed during PMWs run. 2. Controlled by SW, so process delay must be considered |
| 3 | 1. Pulse width or delay can be changed during PMWs run. 2. Controlled by SW, but process delay can be ignored | 1. Concept is complex and not intuitive 2. Deep consideration is required to handle process 3. Two interrupts are required every period of PWM |
| 4 | 1. Pulse width or delay can be changed during PMWs run. | 1. Concept is complex and not intuitive 2. Deep consideration is required to handle process |

Resource Required

| Method | Resources Required |
| --- | --- |
| 1 | 2 Timers and 2 GPIOs |
| 2 | 2 Timers and 2 GPIOs |
| 3 | 1 Timer and 2 GPIOs |
| 4 | 1 Timer and 2 GPIOs |

Process Delay Time and Power Consumption

| Method | Process Delay (us) | Power Consumption (AVG mA) |
| --- | --- | --- |
| 1 | 0 | 20.38 |
| 2 | 2.5 | 18.03 |
| 3 | 0 | 19.92 |
| 4 | 0 | 19.91 |

※ 1 LED is used

Reference – Method 1

Reference – Method 2

Reference – Method 3

Reference – Method 4